The COM port Multiplexor

Beginning with version 2.3g, The $25 Network is better suited to

expansion to three or more nodes.  Up until now you had to specify a separate

interrupt line for each COM port you were using in one machine.  That meant the

network on the HUB computer needed both IRQ4 and IRQ3, which are the normal COM

port interrupt lines.  If you were using a modem or mouse then these also

needed an IRQ line.  Interrupt lines are a scarse resource on a PC so expansion

to 3 nodes was hard enough.  Expansion to four nodes was usually out of the

question.  Now we have solved this problem.

MUX.COM in the UTILS subdirectory shares one IRQ line with up to four

networked COM ports.  To do this you must specify the same interrupt number on

each NET$25 command line, and must enter a new parameter which tells each COM

port to place it's interrupt line in an inactive state.  Then, AFTER the

network, run MUX.COM once, specifying the same interrupt and com ports.  For

example, to use COM1 and COM2 for the network sharing IRQ4, enter the following

in the AUTOEXEC.BAT file:

NET$25 COM1 INT12 OUT12:0

NET$25 COM2 INT12 OUT12:0

NETMUX INT12 COM1 COM2

Remember that the network needs to know the CPU's interrupt number, not the

hardware IRQ number.  To convert from IRQ to INT, add 8 to IRQ  (this works as

long as IRQ is 0,1,2,3,4,5,6, or 7).

The format for NETMUX is:

NETMUX [INTx] [COMa] [COMb] [COMc] [COMd]

Up to four COM ports may be specified.  The INT number is assumed to be 12 if

not specified.  Be sure that NETMUX is run after all the NET$25 lines.  Also,

NETMUX should not be used if fewer than two COM ports are sharing an interrupt.

This multiplex program is designed to run with The $25 Network, not alien

programs or device drivers.  While this may be possible, it may require a small

modification to the alien program, or a small custom program to trick NETMUX

into thinking it is a network program.