�������������������������������������������������������ͻ

�                 ****   NOTE   ****                    �

�                                                       �

�  These programs are included as utilities to The $25  �

�  Network which is a copyrighted, commercial program.  �

�  Nothing on this disk is shareware or  public domain  �

�  and  may  not  be  copied and distributed to anyone  �

�  without the written permission of Information Modes. �

�������������������������������������������������������ͼ

COMIRQ.COM

This utility allows you to turn on or turn off the interrupt request

line (IRQ) on a specific COM port.  The PC/XT/AT hardware was not designed to

permit interrupt lines to be shared.  Since COM1 and COM3 share the same

interrupt, and COM2 and COM4 share the other communications interrupt,  it is

possible for a COM port to get in the way of another COM port's use of its IRQ

line.  COMIRQ will disable (tri-state) a COM port's IRQ line to permit another

COM port to use it.  One likely use is with PROCOMM.  If you have the network

running on COM3 and then load PROCOMM on COM1 you may find either PROCOMM does

not work or the network does not work when you exit PROCOMM.  Run COMIRQ to

disable the network's COM port IRQ line before loading PROCOMM, then disable

PROCOMM's COM port when you exit it, remembering to re-enable the network's COM

port.

EXAMPLE OF USE:

---------------

COMIRQ COM3 IRQ=OFF

or,

COMIRQ COM1 IRQ=ON

or,

COMIRQ COM:3E8 IRQ=OFF

or,

COMIRQ COM:3E8 IRQOFF

or,

COMIRQ COM:3E8 IRQON

etc...