�������������������������������������������������������ͻ

�                 ****   NOTE   ****                    �

�                                                       �

�  These programs are included as utilities to The $25  �

�  Network which is a copyrighted, commercial program.  �

�  Nothing on this disk is shareware or  public domain  �

�  and  may  not  be  copied and distributed to anyone  �

�  without the written permission of Information Modes. �

�������������������������������������������������������ͼ

Documentation for NMOUSE.COM

----------------------------

If you have a mouse you may want to use this utility.

One shortcoming of the original PC hardware design was the lack of

interrupt request lines (IRQs).  Only two interrupt lines were allocated to the

serial ports.  This means that if you have 4 COM ports you must share 2

interrupt lines between 4 COM ports.  But the bus was not designed to support

interrupt sharing.  Fortunately software can sometimes solve the problem, as

with our NETMUX program.

NETMUX was designed to use Information Modes programs, not foreign

programs such as a mouse driver.  NETMUX will not share an interrupt between a

mouse COM port and a network COM port.  So we wrote NMOUSE to make your mouse

driver look like a Information Modes product.

Let's say you have a mouse on COM1, and a modem on COM2.  You just

purchased   The $25 Network  and want to install it on COM3.  The problem is

that COM1 (the mouse) and COM3 (the network) both use IRQ4, and both programs

need exclusive control of that interrupt request line.  The NETMUX program was

designed to solve the problem, but only for two copies of the network, not for

mouse drivers.  So what you can do is trick NETMUX by using NMOUSE.  NMOUSE

makes your mouse driver look like a network driver, so NETMUX is now perfectly

happy.

INSTALLATION

-------------

Load your mouse driver.  Use a CONFIG.SYS driver if possible.

Sometimes you can load a mouse driver as a COM file, but use the CONFIG.SYS

driver if one is supplied.  Then, in AUTOEXEC.BAT add:

NMOUSE COM1 INT12

This example assumes your mouse is on COM1 and is using IRQ4.  The INT12 is the

IRQ number plus 8, so IRQ4=INT12, and IRQ3=INT11.  This line must be placed

after your mouse driver loads, and before the network NET$25 files load.  All

CONFIG.SYS drivers load before AUTOEXEC.BAT programs.  The NET$25 line(s) which

shares the same IRQ must indicate it is using the same INT, and must use the

OUT12 parameter:

NET$25 COM3 INT12 OUT12:0  <plus other parameters>

Then run the multiplexer:

NETMUX INT12 COM1 COM3

Now both COM1 and COM3 are sharing INT12 (IRQ4).  Your mouse should work

independent of the network, and vice versa.

The NETMUX/NMOUSE combination can support more than two COM ports, so

you could add another network COM port to the above example.

SYNTAX:  NMOUSE COMx INTy

x = 1..8

y = 8..15

NOTE:

-----

This NMOUSE program may not work with all mouse drivers out there.  It also may

not work with all mouse driven programs either.  For example, some versions of

Microsoft Windows are extremely rude in that they trap your mouse driver's

interrupt, use their own internal driver (as if your's is incompetent or

something) and then doesn't even restore your driver to it's original state.

This both brings down your driver and our network.