@(#)slowclk.doc 1.2

Using the HDS-300 at Slow Clock Speeds

======================================

By Peter S. Gilmour, Motorola Development Systems, Austin, TX

Using the HDS-300 at slow clock speeds can be broken into two separate

discussions; (1) running the HDS-300 pod at slow clock speeds and (2)

running the target system at slow clock speeds.  In either case, this is

dependent on the processor type/pod in use.  Please consult your Emulator

Module User's Manual and the processor's data sheets for specific details.

Some CMOS processors operate down to DC levels, while other processors do

not operate below 8 MHz.

1. Running the HDS-300 pod at slow clock speeds

The HDS-300 pod uses an emulation processor in background mode to handle

all function requests, i.e. display memory, set/clear breakpoints, start

emulation, etc.  The HDS-300 issues a command to the pod and waits a

fixed amount of time for the pod to respond before reporting no response,

i.e. a "dead" pod.  When the pod clock speed is lowered, it takes longer

for each function request to complete.  At some lower speed, those

functions which take the longest will exceed this timeout value.  The

first functions to fail are usually as follows;

NOBR            Must clear all of breakpoint memory if at least one

breakpoint is set.  Clearing individual breakpoints

still works.

MAP             Single row maps a large amount of emulation memory.

Use more rows with smaller amounts of memory each.

MD/MS/BF/BM/BS  Large amounts of memory.  Use smaller amounts of

memory and repeat the command.

You can move the clock speed jumper "on the fly", i.e. with the emulator

powered up, as long as you always issue a RESET command immediately

afterwards.

2. Running the target system at slow clock speeds

Since all true HDS-300 pods use an internal clock for background activity

and only switch to the target clock for foreground, i.e. target, accesses,

there is no problem for this case.  Just be sure you select the target

clock in your ESIG screen.

It should be noted that the M68HC05C4 pod is a hybrid HDS-200/HDS-300 pod

and does not use an internal clock for background activity.  The lowest

external clock frequency tested was 32 KHz which results in a bus speed of

16 KHz for this pod.

References

----------

1. Section 6.3.g, "Customer Letter for Standalone HDS-300 M68HC05C4/C8

Software, Version 1.0", Motorola part number M68HC05C4HM3/L2.

**EOF**