(***************************************************************)

(*                                                             *)

(*                 ANGLE CLOCK SYSTEM                          *)

(*                                                             *)

(*                                                             *)

(*                                                             *)

(*     DCPM -- DEGREE CLOCK DRIVER AND PERIOD MEASUREMENT      *)

(*                                                             *)

(*                                                             *)

(*                                                             *)

(*    Written  10- 7-91      by Mike Pauwels                   *)

(*    Revised   1-31-92                                        *)

(*                                                             *)

(*                                                             *)

(* ACTION: 1. MEASURE PERIOD OF A TOOTH SIGNAL AND ADVANCE     *)

(*         TCR2 TO THE NEXT MODULUS NUMBER.                    *)

(*         2. INCREMENT TCR2 WITH FINE TOOTH COUNTS AT A RATE  *)

(*         PROPORTIONAL TO THE PREVIOUS PERIOD.                *)

(*         3. REQUEST A CPU INTERRUPT WHEN PERIOD EXCEEDS      *)

(*         A SELECTED MULTIPLE OF THE PREVIOUS PERIOD.         *)

(*         4. ACCEPT SYNCHRONIZATION FROM LINK OR HSR.         *)

(*                                                             *)

(*                                                             *)

(*                                                             *)

(***************************************************************)

FUNCTION DESCRIPTION

DCPM measures the period of an input signal, and drives TCR2 at an

integral multiple of that frequency.  DCPM can be used to detect

the speed of a rotating machine by triggering from a toothed wheel,

counting each tooth as a selected angle increment, and injecting

additional count pulses between the teeth by extrapolating a

sub-multiple of the period from the previous tooth signals.  The

injected counts can be used to trigger other channel events at

proportional positions between actual position signals.

For example, using a wheel with teeth at ten degree increments,

DCPM would advance the count of the teeth by incrementing TCR2 to

the next modulus ten count at each tooth, and incrementing by one

each tenth of a period following to provide a one-degree count to

trigger other functions.

During steady-state operation, DCPM can be programmed to increment

TCR2 at equal intervals between the incoming teeth.  If the period

decreases suddenly during acceleration, it is possible that a tooth

will be detected before all of the inserted counts have been

completed.  DCPM will correct the count to the next modulus number,

and events set to trigger off the missing counts will be initiated

by the greater-than feature of the channel comparators.

In the event of sudden increase of the tooth period during

deceleration, the number of inserted counts is limited to one less

than the modulus number, so that the last interval will represent

the increase in the period, but no extra intervals will be

reflected in TCR2.

DCPM continues to track the number of intervals between teeth, and

compares this count to a limit parameter.  If the number of

intervals exceeds this parameter, an interrupt is requested.  This

feature provides a stall indication when the tooth signal stops.

The period is written coherently to a 24-bit parameter.  The

resolution is in terms of TCR1.  Note that the period value is not

valid until the second tooth is detected.

The maximum and minimum periods of the input signal are limited by

timing considerations in the TPU and the range of values processed.

The maximum period (minimum speed) value is determined by:

Period_max = TCR1_resolution (microsec) * 65536 * 4

Rotational_speed = 60,000,000 / (Period_max * Teeth_per_rev)

For a TCR1 resolution of .238 microseconds per count, the maximum

period is 62,500 microseconds per tooth.  Given a 36 tooth wheel,

this period corresponds to a rotational speed of 26.7 RPM.

The minimum period of the input signal is a system decision,

determined by the desired loading of the TPU.  When an input

transition occurs indicating a real tooth detection, the function

executes a service routine to measure the period, update the TCR2,

and setup for a match event for the first fine tooth.  This routine

requires approximately 9 microseconds to execute including 10

system clocks for service request response time.

At each degree clock period, a service request is generated.  The

TPU requires a minimum of 10 clock cycles to respond to request,

and the service routine executes in another 18 cycles.  Thus each

degree count requires 1.67 microseconds (at 16.77MHz.)

The total time to execute the extrapolation phase and the count

phases for a machine with 36 teeth rotating at 8000 RPM and 9

degree counts per tooth would be 24 microseconds.  Since the

time for the machine to travel 10 degrees at 8000 RPM would be:

60 Sec/Min / 36 Teeth/Rev * 8000 Rev/Min = 208 microsec.

The TPU load would be 208 microsec/ 24 microsec =  8.66%.

The count of the teeth is maintained in the TCR2 counter, and other

channels can be made to trigger off the degree position.  An

optional feature of DCPM provides for the count to progress from 0

to the maximum degree count then in one step to $8000, and again by

steps to $8000 plus the maximum count.  This feature allows DCPM to

be compatible with certain periodicly triggered functions.

The function requires a synchronization request from either the

host of from another channel.  Until this request is received, TCR2

will idle at a negative number, and a host interrupt will be

requested at each tooth.  Synchronization is not accepted until

after the first tooth is received.

Synchronization by a Link from another channel results in a sync

flag being set.  It is assumed that the other channel has directly

updated the degree parameter, and the TCR2 count proceeds with the

newly synchronized count at the detection of the next tooth edge.

If synchronization is requested from the host, a buffer parameter

is used to assure coherent writes to the angle parameter.

(*                                                                    *)

(*DATA STRUCTURE                                                      *)

(*                                                                    *)

(* NOTE: MUST BE USED ON CH 14 or 15.                                 *)

(*                                                                    *)

(* PARM   NAME              BY                  DESCRIPTION           *)

(*                                                                    *)

(*  0   CONFIG             CPU    Selects Rising/Falling Tooth edge   *)

(*  0   REF_TIME           TPU    Temporary TCR1 value                *)

(*                                                                    *)

(*  1H  TOOTH              TPU    Tooth Count                         *)

(*  1L  DEGREE             TPU    Degree Count between Teeth          *)

(*  1H  LINK_TOOTH         TPU    Force Value for TOOTH with Link     *)

(*                                                                    *)

(*  2H  MODULUS            CPU    TCR2 Counts per tooth               *)

(*  2L  STALL_COUNT        CPU    DEGREE Count for Interrupt          *)

(*                                                                    *)

(*  3H  NUMBER_TEETH       CPU    Teeth per cycle                     *)

(*  3L  RATIO              CPU    Period Multiplier                   *)

(*    Note: Degree Period = PERIOD * RATIO / 1024                     *)

(*                                                                    *)

(*  4L  PERIOD_MSB         TPU    Tooth period MS byte                *)

(*                                                                    *)

(*  5   PERIOD_LSW         TPU    Least significant word              *)

(*                                                                    *)

(*  6   INTERPOLATE        TPU    Time Between Degrees                *)

(*                                                                    *)

(*  7H  PHASE              TPU    Phase of TCR2                       *)

(*  7L  WRITE_TOOTH        CPU    Force Value for TOOTH and TCR2      *)

(*                                                                    *)

(*SEQ0  TWO_PHASE           Indicates two-phase sequence              *)

(*                                                                    *)

(*HSR11 INIT                Initialize                                *)

(*                                                                    *)

(*HSR10 SET                 Write TOOTH and Start Fill                *)

(*                                                                    *)

(*LINK  LINK_SET            Write TOOTH and Start Fill                *)

(*                                                                    *)

(*No LINK Generated                                                   *)

(*                                                                    *)

(*IRQ   Interrupt on every Tooth until Syncronization and on STALL    *)

(*                                                                    *)

(*                                                                    *)