READ / WRITE TIMERS & PIN TPU FUNCTION (RWTPIN)

===============================================

DESCRIPTION

------------

The RWTPIN TPU function enables the CPU to read both the TCR1 and TCR2 timer

counters via locations in PRAM and alternatively, to selectively load TCR1 or

2 with a CPU supplied value contained in PRAM.  The function also optionally

allows the user to control the pin state and direction of the RWTPIN channel.

A pinstate parameter is maintained in PRAM and is updated on every service

request to contain one of two values representing the current pinstate

regardless of whether the pin is programmed as an input or output ($8000 = pin

high, $0000 = pin low).

The function also receives links, and on receipt of a link will read the two

TCRs into PRAM and update the pinstate parameter. After all operations, a

(maskable) interrupt request to the CPU is generated.

Effectively, this function allows the CPU to control the channel pin, the

channel pin and the TCRs or just the TCRs.  To control the pin only, the the

'read TCR' option is used and the values returned ignored.  Being able to

control the TCRs without effect on the pin allows this function to be run on a

TPU channel whose pin is being controlled by a different function running on

another channel such e.g. in the case of a slave stepper motor channel.

FUNCTION CODE SIZE

------------------

The code size of a TPU function is important when assessing which combination

of functions can fit into a given microcode space in ROM or emulation RAM. The

code size of the RWTPIN function is:-  10 �instructions + 8 entries =  18 long

words total.

CONTROL BITS

------------

HSR1   HSR0         Action

----   ----         ------

0       0          No action.

0       1          Read TCRs & read/write pin

1       0          Write TCR1, read TCRs & read/write pin

1       1          Write TCR2, read TCRs & read/write pin

HSQ1   HSQ0         Action

----   ----         ------

X       X          Not used.

Links Accepted: YES             Links Generated: NO

Interrupts Generated After:     All States.

PARAMETERS

-----------

The following sections describe the parameters used in the RWTPIN function,

the location and size of each parameter being given after the name.

CHAN_CONTROL            Parameter 2, bits 0..8

This CPU written parameter is used to setup the direction and level of the

RWTPIN channel pin.  It uses the standard channel control format used by other

TPU functions and shown in table 2-3 of the TPU ref. manual.  Note that since

there are no match or transition operations in this function then the

selection of the timebase via TBS and pin action via PAC are irrelevant.  The

important fields are the channel direction control via TBS and the pin state

control via PSC.  It is possible to use the RWTPIN function and not control

the channel pin, this is accomplished by setting the CHAN_CONTROL parameter to

%111111111.

CHAN_PINSTATE           Parameter 3, bits 0..15

This TPU written parameter is used to maintain a value representing the

current pin level.  When the RWTPIN channel is serviced, the TPU reads the

pinstate and stores $8000 in CHAN_PINSTATE if the the pin was high and $0000

if the pin was low.  This operation occurs regardless of whether the channel

is programmed as an input or output.

TCR1_VAL                Parameter 4, bits 0..15

This parameter serves two purposes and can be written by either the CPU, TPU

or both as follows.  When a load of TCR1 is desired then TCR1_VAL contains the

CPU written value that will be loaded into TCR1 when the host service request

is executed.  As part of this service, TCR1 is also read by the TPU and the

value written back into TCR1_VAL.

When a 'TCR read' host service request or link service request is serviced,

then TCR1_VAL is written by the TPU with the latest value of TCR1.

TCR2_VAL                Parameter 5, bits 0..15

This parameter serves two purposes and can be written by either the CPU, TPU

or both as follows.  When a load of TCR2 is desired then TCR2_VAL contains the

CPU written value that will be loaded into TCR2 when the host service request

is executed.  As part of this service, TCR2 is also read by the TPU and the

value written back into TCR2_VAL.

When a 'TCR read' host service request or link service request is serviced,

then TCR2_VAL is written by the TPU with the latest value of TCR2.

HOST INITIALISATION

-------------------

The CPU initialises the RWTPIN function by the following means:-

1) Disabling the channel by clearing the two channel priority bits.

2) Selecting the RWTPIN function on the channel by writing the RWTPIN function

number to the function select bits.

3) Writing CHAN_CONTROL in the channel parameter RAM.

4) Writing TCR1_VAL or TCR2_VAL if a TCR write is to be made.

5)  Issuing a HSR %01 to setup the pin and read the two TCRs, or, issuing a

HSR %1X to write one of the two TCRS, setup the pin and then read both TCRs .

6)  Enabling service by assigning H,M or L priority to the channel via the two

channel priority bits.

The TPU then executes the selected host state and asserts an interrupt request

when finished.

NOTES ON USE AND PERFORMANCE OF RWTPIN FUNCTION

------------------------------------------------

PERFORMANCE:  Like all TPU functions, the performance limit of the RWTPIN

function in a given application is largely dependent on the activity on other

TPU channels.  This is due to the operational nature of the scheduler.  Where

a single RWTPIN channel is being used and no other TPU channels are active,

the minimum time between host service requests is 24 CPU clock cycles for

reading the TCRs and 28 CPU clock cycles for writing a TCR.  When more TPU

channels are active, this performance will generally be degraded.  However,

the scheduler assures that the worst case latences in any TPU application can

be calculated, so it is recommended that the guidelines given in the TPU

reference manual are used to perform an analysis on any proposed TPU

application that appears to approach the performance limits of the TPU.

INTERRUPT TIMING:  For maximum TPU code efficiency, the interrupt request from

the TPU to the CPU is not asserted exactly at the end of the state.  This

means that there is a delay of 8 CPU clocks from the assertion of the

interrupt request to the new pinstate data being valid.

RETURNED VALUE WHEN WRITING A TCR:  As part of the service of a TCR write, the

TPU also reads both TCRs and returns their values in the TCR1_VAL  and

TCR2_VAL parameters. The   execution time of the TPU code means that there is

a delay between the TCR write and read of 4 or 8 CPU clocks (TCR1 or 2) which

may result in a difference between the initial CPU written and TPU returned

TCRx_VAL depending on the prescaler setting for TCRx.

USING RWTPIN FOR PIN CONTROL ONLY:   This is achieved by using the host

service request to read the TCRs (%01).  Since reading the TCRs does not

affect them in any way, the pin can be controlled via CHAN_CONTROL and the

results in TCR1_VAL and TCR2_VAL ignored.

USING RWTPIN FOR TCR CONTROL ONLY:   The user may wish to read or write the

TCRs without affecting the channel pin (which may then be controlled by

another channel if required), this is achieved by issuing the appropriate HSR

for the TCR action required, but with CHAN_CONTROL = %111111111.

WRITING ONE TCR AND READING THE OTHER:  This operation is automatic:- when one

TCR is written then both it and the other are read back into TCR1_VAL and

TCR2_VAL.

WRITING BOTH TCRs:  This is a two stage operation.  The user should load the

desired value for one of the TCRs into PRAM and issue the appropriate HSR to

write it. When the first service is complete then the second TCR value can be

written into PRAM and the other 'write' HSR issued.  Note the order here, if

the two values were written before issuing the first HSR then the second value

would be overwritten by the read of the TCRs performed as part of the first

service.

LINKS: Since the link request flag is negated in all states, then a link that

is received just before or possibly during the execution of a host service

request may be negated without being serviced.  In practice, this will not be

a problem as all the other states are supersets of the link state.  It should

be remembered that the link state (RWTPIN_LNK) also configures the channel via

CHAN_CONTROL, this has two implications:

i) If a regular link from another channel is being used to read the TCRs, then

the pin can be controlled without using host service requests. The new pin

configuration can simply be written into the CHAN_CONTROL parameter and the

pin will be automatically updated on the next link service.

ii) If a regular link from another channel is being used to read the TCRs and

precise CPU control is required on the timing of a pin state change, then it

may be necessary to temporarily disable the RWTPIN channel (or suspend links)

to avoid the new pin state being inadvertently set by the link state before

the CPU issues the appropriate HSR.