UNIVERSAL PROGRAMMER AND TESTER EXPRO-60/80

README.DOC

**********

1. EXPRO-60/80 has consolidated its with coloured pull-down menu driver

software into 2 diskettes,

First diskette (#1) consists of the following files:

BPPGM.BIN   : Device driver file for BIPOLAR PROM.

DEVICE.LST  : EXPRO-60/80 can program Device's list.

EEP1.BIN    : Device driver file for EEPROM.

EEP1P.BIN   : Device driver file for EEPROM with LINK-P1.

EPLD1.BIN   : Device driver file for ALTERA,TI,INTEL EPLD.

EPP1024.BIN : Device driver file for EPROM over 1 M bits.

EPP1024P.BIN: Device driver file for EPROM over 1 M bits with LINK-P1.

EPP512.BIN  : Device driver file for EPPROM under 512K bits.

EXPRO.EXE   : Auto device driver file.

EXPRO.INF   : Device information list.

GAL1.BIN    : Device driver file for LATTICE,NS GAL.

GAL2.BIN    : Device driver file for AMD PALCE Series.

GAL3.BIN    : Device driver file for LATTICE,NS GAL6001.

INSTALL.EXE : Installation diskette #1,#2 to HARD DISK.

MPU1.BIN    : Device driver file for HITACHI 63701V0,TI 7742,77C82.

PALPx.BIN   : Device driver file for PAL device.

x:from 1 to 5.

PEEL1.BIN   : Device driver file for ICT PEEL.

PEEL2.BIN   : Device driver file for ICT PEEL.

PGM48.BIN   : Device driver file for 8748 series MPU.

PGM51.BIN   : Device driver file for 8751 series MPU.

PGMZ8.BIN   : Device driver file for Z8 series MPU.

README.DOC  : This document.

SEEP1.BIN   : Device driver file for 8 pin serial EEPROM and special

serial PROM.

SEEP2.BIN   : Device driver file for 8 pin serial EEPROM and special

serial PROM.

Second diskette (#2) consists of the following files:

A26CV12.BIN : Device driver file for AMD PALCE26V12/24V10,

LATTICE GAL26CV12.

A63705.BIN  : Device driver file for HITACHI 63706.

APIC16.BIN  : Device driver file for MICROCHIP PIC16C54/55/56/57.

APIC71.BIN  : Device driver file for MICROCHIP PIC16C71.

AZ86E08.BIN : Device driver file for ZILOG Z86E08.

CMOS40.LIB  : CMOS 40 series library.

CMOS45.LIB  : CMOS 45 series library.

DISASM <DIR>:Subdirectroy of disassembler files includes.

DASM09.EXE  : Disassembler for 6809

DASM11.EXE  : Disassembler for 68HC11

DASM48.EXE  : Disassembler for 8048

DASM51.EXE  : Disassembler for 8051

DASM85.EXE  : Disassembler for 8085

DASMZ80.EXE : Disassembler for Z8

DRAMTEST.EXE: Test dynamic RAM.

EPLD2.BIN   : Device driver file for INTEL EPLD.

EEP2.BIN    : Device driver file for EEPROM.

FPLP1.BIN   : Device driver file for SIGNETICS FPL device.

FPLP2.BIN   : Device driver file for SIGNETICS FPL device.

LOGTEST.EXE : Test TTL74,COMS40/45 series.

PALTEST.EXE : PLD VECTOR TESTING program.

PRODUCTS    : Introduction of all SUNSHINE products.

SRAMTEST.EXE: Test static RAM.

TESTPRO.EXE : Test EXPRO-60/80 hardware.

TTL74.LIB   : TTL 74 series library.

VECTEDIT.EXE: Edit and test VECTOR.

UTIL <DIR>  : Subdirectroy of UTILITY files includes.

DUMP.EXE    : Dump file to Console in BINARY format.

EDBIN.EXE   : Edit a binary file.

FILEDIV.EXE : Divide a file into couple files

FILSHUFF.EXE: Assemble couple files into one.

HEXBIN.EXE  : HEX in BINARY file converter.

HEXBIN2.EXE : EXTENDED HEX to BINARY converter.

SEP2MB.EXE  : Seperate a BINARY File large than 2Mbits

SPLIT2.EXE  : 2-way file splitter.

SPILT4.EXE  : 4-way file splitter.

SHUFF2.EXE  : 2-way file shuffler.

SHUFF4.EXE  : 4-way file ahuffler.

Note: EXPRO-60/80 software was released on Sept 1992, all EXPRO-60

software before this date is not compatible with present

software. Please do not mix them up.

2. To enhance EXPRO-60/80's software function and make it easier to

operate, great amendments were made on the software driver. In

addition to altering into coloured version, other changes are:

a) pull-down menu driver furnishes user with simple operating

environment

b) detailed status field display provides user with complete IC

programming information, work path and current working file

c) all functions can be operated under main-menu providing user

with continuous operating procedures

d) equipped with Macrofile function which greatly simplifies user's

operating procedures

3. INSTALLATION

This segment describes the method of installing an UNIVERSAL

programmer on an IBM PC/XT/AT/386/486 computer or compatible

running MS-DOS or PC-DOS.

It is assumed that the installer is familiar with the installation

of PC add-on cards and software in PCs running MS-DOS.

3.1 Host System Requirements

*IBM PC/XT/AT/386/486 or compatible PC.

*Min. 640K bytes memory.

*Min. One floppy disk drive. A hard disk is preferable.

*Operation system :MS-DOS or PC-DOS ,version 2.0 or later.

3.2 Software Installation Procedures

Running the INSTALL.EXE to setup EXPRO-60/80's S/W to HARD disk.

Running EXPRO.EXE will start the system when installation is completed.

4. About GANG Adaptors using

ADP-GAL16V18,   ADP-GAL20V8,    ADP-CE16V8, ADP-CE20V8,  ADP-N3001,

ADP-PEEL18CV8,  ADP-PEEL22CV10, ADP-EP32,   ADP-EP40,    ADP-EP32A-PLCC,

ADP-EP32B-PLCC, ADP-EP40-PLCC,  ADP-51F,    ADP-48F.

The adaptors listed above are GANG adaptors (can program 4 devices

simultaneously). These GANG adaptors' driver program (all in Adaptor

#3 diskette) needs to be independently executed and cannot be called

through EXPRO.EXE. Therefore, the GANG adaptor driver file does not need

to be in the same directory as EXPRO.EXE while non-GANG adaptor driver

file must be in the same directory as EXPRO.EXE to be conveniently called

through EXPRO.EXE.

5. Revision of EXPRO.EXE

Version V7.00: On Sept 1, 1992, EXPRO-80 hardware and software were

formally released. Software is compatible with

EXPRO-60 hardware and will automatically identify

between EXPRO-60 and EXPRO-80 hardware.

Due to the restriction of DOS, our software can only

allocate memory buffer to the maximum of 256K bytes

(2 Mbits), thus we have in the "RunFunc" menu,

"Copy file to EPROM" and "Read EPROM to file" these

two functions to solve the problem of insufficient

memory buffer.

Below is its operating procedure:

"Read EPROM to file" - Software will require you to

input filename, then display "Ready to Read (Y/<CR>)?".

If you press 'Y', software will read EPROM's content on

the textool and save it into diskette.

"Copy file to EPROM" - Software will require you to select

the file to be copied to the EPROM file. After selection,

software will inquire "Ready to program (Y/<CR>)?".

When 'Y' is pressed, software will read the file's

content and program it to EPROM.

Version V7.10: On Nov 1,1992, software was updated with function of

executing adaptor with driver program. If you happen

to wish to program a Device that requires an adaptor,

the right bottom corner of Status field will display

"ADAPTOR: adaptor's module name" to remind you the

need of an adaptor in order to program.

Version V7.13: On Nov 20, 1992, to simplify user's operating procedure,

"Macro setup" and "Run Macro" were included in Misc.'s menu.

Briefly, MACRO's function in 'Misc.' are (1) Macro setup

(2) Run Macro,

Step 1: After selecting "Macro setup", the bottom corner

of status field displays " [Macro Mode]:<F9> to

abort, <F10> to save". Every operating procedure

will be recorded by the software till you press

<F10>. Software will then require you to input

Macro filename, after entering the filename,

"[Macro Mode]:..." will then vanish.

Step 2: After selecting "Run Macro", software will

require you to input the Macro file to be

executed. "Macro processing" will appear on

the bottom corner of status field after selecting

Macro file till execution is completed.

Version V7.15: On Dec 17, 1992, for the convenience of user, software

was updated to classify IC Test into the following:

1. Logic IC test     : Test TTL74 , CMOS40/45 series

2. Dynamic RAM test  : Test Dynamic RAM

3. Static RAM test   : Test Static RAM

4. User vector editor: User can edit Vector and test

ICs that we did not include in

our Type. Modes selectable are

8, 14, 16, 20, 24, 28, 32 and 40

pins.

Version V7.22: On Mar 23, 1993, EXPRO /M=MACROfilename

If you want to run MACRO file after entering

EXPRO.EXE program, you could input "EXPRO /M=MARCOFILENAME"

at DOS prompt.

version V8.00 : May 31, 1993

EXPRO.EXE V8.00 has the following modifications:

a. Software V8.00 can be used with either SAC-101A, System Adaptor Card or

LINK-P1, Printer Port Adaptor Module to control EXPRO-60/80 hardware.

The software automatically detects the kind of interface used. To

avoid confusions with old versions, the programming driver files' file

extention name EXE has been changed to BIN.

b. V8.00 and later versions simplify Read/Program of high-capacity EPROM.

When user selects EPROM size larger than software allocated memory

buffer, software opens a file (EDBIN.MEM) with same EPROM size

automatically in the "Work path" to use file buffer when software

access EPROM's data.

Example:

Assumption: software allocated memory buffer space is 256K bytes

Work path: D:\EXPRO\

When programming 27C040 (512 Kbytes), allocated memory buffer becomes

insufficient. Software automatically opens a 512 Kbytes file

(D:\EXPRO\EDBIN.MEM). If file opening is successful, "EDBIN.MEM" will

appear after "Buffer address :" inside the status field. After this,

the user may change work path's directory and not affect EDBIN.MEM

access.

Upon exiting the main program, D:\EXPRO\EDBIN.MEM is recorded in

EXPRO.DAT for future use. As not to occupy disk space,

D:\EXPRO\EDBIN.MEM is deleted.

NOTE: For executing the above, take note of the following:

a. If the PC's memory RAM is sufficient, it is suggested that the

excess memory RAM be set into RAM DISK (refer to DOS manual).

This way, the maximum speed can be attained during Read/Program

EPROM and avoid hard disk Read/Write frequency.

b. If the PC has insufficient RAM, it is suggested that the biggest free

space of hard disk of faster access time be used as not to affect the

speed of Read/Program EPROM.

c. If the disk space is insufficient or Read/Write is not possible,

"Disk-Buffer access error. Refer to README.DOC" would appear on the

screen. At this point, determine if there is sufficient buffer to

access EPROM data based on the length of the "Buf start:","Buf end:"

and "Dev start:","Dev end:" inside  the status field. If buffer is

insufficient, solve the above problem on the disk before reentering

main program.

6.Revision of DRIVER FILE

����������������������������������������������������������������������������Ŀ

� A75P008.BIN                                                                �

����������������������������������������������������������������������������Ĵ

�   Version V8.01 : On 9/3/93 			                             �

�                   Addition : NEC 	uPD75P516GF,uPD75P518GF   	     �

�   NOTE : Adaptor ADP-uPD75P516/8 is required.                              �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� A8796.BIN                                                                  �

����������������������������������������������������������������������������Ĵ

�   Version V7.02 : On 2/15/93 			                             �

�                   Addition : INTEL 87C196KD/20                             �

�                                                                            �

�   NOTE : Adaptor ADP-8796 is required.                                     �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� A68705J2.BIN                                                               �

����������������������������������������������������������������������������Ĵ

�   Version V8.00 : On 5/25/93 first version                                 �

�                   Supports : MOTOROLA MC68HC705J2,MC68HC705P9              �

�                                                                            �

�   NOTE : Adaptor ADP-68HC70J2/P9 is required.                              �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� AEPM7032.BIN                                                               �

����������������������������������������������������������������������������Ĵ

�   Version V7.00 : On 2/17/93 first version                                 �

�                   Supports : ALTERA EPM7032                                �

�                                                                            �

�   NOTE : Adaptor ADP-EPM7032 is required.                                  �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� AEPS464.BIN                                                                �

����������������������������������������������������������������������������Ĵ

�   Version V7.00 : On 2/22/93 first version                                 �

�                   Supports : ALTERA EPS464                                 �

�                                                                            �

�   NOTE : Adaptor ADP-EPM7032 is required.                                  �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� APIC16C71.BIN                                                              �

����������������������������������������������������������������������������Ĵ

�   Version V7.00 : On 3/26/93 first version                                 �

�                   Supports : MICROCHIP PIC16C71                            �

�                                                                            �

� NOTE : The following corrections are necessary for adaptor ADP-PIC16       �

�        purchased before 31/3/93 to program PIC16C71 (No adaptor is         �

�        required with EXPRO-80).                                            �

�                                                                            �

�        1. Connect a short wire between P18 of U1 and P29 of U3.            �

�        2. Connect a 1.5K ohms resistor between P7 of U3 and P17 of U3.     �

�        2. Connect a 1.5K ohms resistor between P7 of U3 and P18 of U3.     �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� APIC17.BIN								     �

����������������������������������������������������������������������������Ĵ

�   Version V8.00 : On 5/25/93 first version                                 �

�                   Supports : MICORCHIP PIC17C42			     �

�                                                                            �

�   NOTE : Adaptor ADP-PIC17 is required.				     �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� APSD301.BIN                                                                �

����������������������������������������������������������������������������Ĵ

�   Version V7.01 : On 4/19/93 			                             �

�                   Addition : WSI PSD301/C1,PSD311/C1,PSD301L,PSD311L       �

�                                                                            �

�   NOTE : Adaptor ADP-PSD301 is required.                                   �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� APSD302.BIN                                                                �

����������������������������������������������������������������������������Ĵ

�   Version V7.01 : On 4/19/93 			                             �

�                   Addition : WSI PSD302/C1,PSD312/C1,PSD302L,PSD312L       �

�                                  PSD303/C1,PSD313/C1,PSD303L,PSD313L       �

�   NOTE : Adaptor ADP-PSD301 is required.                                   �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� BPPGM.BIN                                                                  �

����������������������������������������������������������������������������Ĵ

�   Version V8.01 : On 6/15/93 			                             �

�                   Addition : ATMEL 27HC641R,27HC642R			     �

�                                                                            �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� BPPGM1.BIN                                                                 �

����������������������������������������������������������������������������Ĵ

�   Version V8.00 : On 9/29/93 			                             �

�                   Addition : WSI WS57C43C,WS57C49C,WS57C71C,WS57C191/291C  �

�                                                                            �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� EEP1.BIN                                                                   �

����������������������������������������������������������������������������Ĵ

�   Version V7.05 : On 4/26/93 			                             �

�                   Addition : ATMEL   AT28MC020,AT28MC040	             �

�                              NEC     uPD28C256			     �

�                              SAMSUNG KM28C64A/65A,KM28C256A,KM29C010	     �

� 			       SEEQ    DQ28HC64,DQ28C65,DQ28C256A,DQ28HC256  �

�                                      DQ28C010,DQM28C010,DQM28C010A	     �

�   Version V8.01 : On 7/14/93 			                             �

�                   Addition : CATALYST CAT28C65A,CAT28C256		     �

�			       DALLAS   DS1220Y				     �

�			       EXEL     XL2804A,XL28C16B,XL28C64,XL28C64B    �

�				        XL2865A,XL28C256		     �

�			       OKI	MSM28C256			     �

�			       XICOR	X28C16,  X28HC16, X28HC64	     �

�					X28HC256,X28VC256,X28TC256	     �

�   Version V8.02 : On 7/23/93 			                             �

�                   Addition : HITACHI HN58C66				     �

�   Version V8.03 : On 7/30/93 			                             �

�                   Addition : SGS-THOMSON M28F256A,M28F512,M28F101,M28F201  �

�					   MKI48Z02/12			     �

�			       MITSUBISHI  M5M28F101P			     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� EEP2.BIN                                                                   �

����������������������������������������������������������������������������Ĵ

�   Version V8.02 : On 7/8/93 			                             �

�                   Addition : HITACHI HN28F4001		             �

�                              MXIC    MX28F1000,MX28F4000		     �

�   Version V8.04 : On 7/23/93 			                             �

�                   Addition : TOSHIBA TC58257AP TC58257AP-LV  		     �

�   Version V8.06 : On 8/6/93 			                             �

�                   Addition : MITSUBISHI  M5M28F102P			     �

�			       SGS-THOMSON M28F102,MK48Z08/18,MK48Z09/19     �

�					   MKI48Z18,MK48Z30/Y,MK48Z32/Y      �

�					   M48Z128/Y,M46Z128/Y,M48Z256/Y     �

�					   M46Z256/Y,M48Z512/Y	   	     �

�   Version V8.07 : On 9/23/93 			                             �

�                   Addition : SST         28EE011			     �

�			       XICOR       XM28C010,XM28C020,XM28C040	     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� EPP1024.BIN/EPP1024P.BIN						     �

����������������������������������������������������������������������������Ĵ

�   Version V8.01 : On 7/7/93 			                             �

�                   Addition : MXIC  MX27C4000				     �

�                              NEC   uPD27C8000                              �

�   Version V8.04 : On 10/19/93						     �

�                   Addition : ATMEL AT27C020 , AT27C080		     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� PALP3.BIN								     �

����������������������������������������������������������������������������Ĵ

�   Version V8.01 : On 7/23/93 			                             �

�                   Addition : CYPRESS PALC22V10D			     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� PGM51.BIN                                                                  �

����������������������������������������������������������������������������Ĵ

�   Version V7.07 : On 2/17/93 			                             �

�                   Addition : DALLAS DS5000-8K,DS5000-32K                   �

�   Version V8.01 : On 7/26/93 			                             �

�                   Addition : ATMEL  AT89C51				     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� PGMZ8.BIN                                                                  �

����������������������������������������������������������������������������Ĵ

�   Version V7.05 : On 2/5/93 			                             �

�                   Addition : ZILOG Z86E23			             �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� PEEL1.BIN 	                                                             �

����������������������������������������������������������������������������Ĵ

�   Version V8.00 : On 5/21/93 			                             �

�                   Addition : ICT PEEL22CV10A,PEEL22CV10A+	             �

�   Version V8.01 : On 7/2/93 			                             �

�                   Addition : NS GAL22CV10-7/10			     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� PEEL2.BIN 	                                                             �

����������������������������������������������������������������������������Ĵ

�   Version V8.00 : On 5/21/93						     �

�                   Addition : ICT PA7024			             �

�   Version V8.01 : On 9/24/93						     �

�                   Addition : ICT PA7128			             �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� SEEP1.BIN								     �

����������������������������������������������������������������������������Ĵ

�   Version V8.01 : On 6/7/93 						     �

�                   Supports : SGS-THOMSOM ST93C56,ST24C01,ST24C08,ST24C16,  �

�					   ST25C01,ST24508		     �

�                              NS    NM24C02/L,NM24C04/L,NM24C08,NM24C16,    �

�                                    NM24C03/L,NM24C05/L,NM24C09,NM24C17     �

�                              SEEQ  2913A/C,2914A,2922A,2934A               �

�                              MICROCHIP 24C16			             �

�   Version V8.02 : On 8/25/93 						     �

�                   Supports : XILINX XC1736D,XC1765D			     �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

����������������������������������������������������������������������������Ŀ

� SEEP2.BIN								     �

����������������������������������������������������������������������������Ĵ

�   Version V7.03 : On 4/3/93 			                             �

�                   Addition : AT&T ATT1736F,ATT1765F,ATT17128F              �

�   Version V8.01 : On 6/10/93 						     �

�                   Supports : SEEQ  2919G,2929G,2939G		             �

�                              SONY  CXK1023M/P,CXK1024/P		     �

�   Version V8.02 : On 8/6/93 						     �

�                   Supports : PHILIPS PCx8581,PCx8582,PCx8594,PCx8594       �

�   NOTE : No adaptor is required.		                             �

������������������������������������������������������������������������������

7. To program EPROM by way of EVEN/ODD ( with driver file EPP1024.BIN

version V7.05 for 1M to 8M or driver file EPP512.BIN version V7.02

for 2716 to 27512 ), you can operate as follows,

EX: PROGRAM 27C256

a. Modify zone

Buffer start : 0

Buffer end   : 3FFF

Device start : 0

When you press 'Even' in the program function, software will execute

program procedure and halt at EPROM address 3FFF.

b. Modify zone

Buffer start : 0

Buffer end   : 7FFF

Device start : 4000

When you press 'Odd' in the program function, software will execute

program procedure and halt at address 7FFF.