Mad Apple Forth v1.0

1 March 1989

Origins

-------

Mad Apple Forth originated under DOS 3.3 from the 6502 Fig-Forth for the Apple.

It was then modified by Ted St. John and Bill O'Ryan to more closely resemble

the Forth-79 standard.  Mad Apple Forth went through many changes during this

time, at times changing so fast you couldn't keep up with the current version!

Then Bill O'Ryan moved away, and Ted St. John stopped working with it to any

degree.  I started supporting and enhancing it about that time.  Up until that

time modifications to Mad Apple Forth had been done using a version of the

target compiler for fig Forth modified by Mr. O'Ryan.  He also tended to be the

only one who could get it to work.

I disassembled Mad Apple Forth into Merlin Assembler source, and then modified

it to work with Prodos through the use of Forth words tacked on top of the DOS

3.3 originals.  Mad Apple Forth was then converted to work with Prodos

intrinsically.

I also added many utilities and enhanced old ones.  I converted many old

utilities from DOS 3.3 to Prodos.  I added a lot of Forth words, including a

new DUMP and DECOMPILE.  Many more words were vectored to allow fancier

programming.  A new "on the fly" decompiler was created.  Support for 3.5"

drives was added along with the ability to change the block/drive allocations

on the fly.

After converting Mad Apple Forth to Prodos, I then added support to the basic

kernal for the Super Serial Card using interrupts and machine code for speed.

Work currently continues on this version and is not yet complete.  Words are

included to allow complete control of Mad Apple Forth over the modem.

The Future

----------

Currently Mad Apple Forth remains in 6502 code.  One future improvement is

therefore to convert MAF to 65802 code and full 16-bit capability.

Specifically, I hope to convert MAF to 65802 code capable of running on any

Apple II+ with a 65802 installed.

After a 65802 conversion multitasking is high on the list.  The multitasking

scheme will be cooperative, and a simple priority scheme will be implemented.

Semaphores will control access to various devices.  The Super Serial Card

support will also be converted to support multiple ports with multiple tasks.

With the 65802 version may come full support as a shareware product, inluding

upgrades and detailed documentation.

File-oriented Forth blocks are also in the works.  Up to four files will

be available at one time.  Using a file will supplant the entire physical

drive system, prevent access to physical drives.

Memory Map

----------

$95FF        LIMIT - top of Forth disk buffers

$91FF              - (for versions containing Forth File access words)

$85EF        FIRST - bottom of Forth disk buffers

$81EF              - (for versions containing Forth File access words)

$2Fxx        PAD - varies... at fixed location relative to HERE

$2Fxx        HERE - varies...

$903         Forth WARM start

$900         Forth COLD start

$900         Start of Forth Kernal

$800         User Area

$3C7         Data stack - grows downward, location fixed

$300         Standard Apple vectors - Reset and control-Y are connected

$200         TIB - terminal input buffer

$100         Stack - the return stack

$000         DP - used for W, UP, and other misc

Note that FIRST is defined as a function of LIMIT and #BUFS - so to

move the buffers it is only necessary to specify the number of

buffers (#BUFS) and the location of the last valid buffer

address (LIMIT).

Revision History

----------------

Original:    Mad Apple Forth begin as figForth.  It was modified heavily by

Bill O'Ryan, with help from Ted St. John, Marty Isaacs, and many

others.  Mad Apple Forth had been modified to approximate the

Forth-79 standard.  Dates were used to identify the most recent

version of Mad Apple Forth.

v1.0:        Mad Apple Forth was disassembled and Merlin assembler source was

created.  MAF was converted to Prodos, and Prodos MLI call support

was implemented.  Version numbers replaced version dates.

v1.1:        A kernal with Super Serial Card support was released for the

first time.  The text comparison word -TEXT was added.  A user

definable bell was added.  One bug - EXPECT will back the cursor

past the left margin after it has beeped once - this is inherent

in the original figForth source.

v2.0:        Planned.  Create a ROMable Forth kernal.  Fix EXPECT.  Continue

expanding Super Serial Card support.  Upgrade source further to

include macros and meaningful labels instead of disassembler-

created data names.

Note that the planned ROMable Forth kernal will by its nature

have a different memory map than the current release.

Word Breakdown

--------------

Stats of Mad Apple Forth: 166 words, 9853 bytes; average, 27 bytes/word

Breakdown (slightly dated):

20  5.46% constants

161 43.98% colon definitions

5  1.36% variables

19  5.19% user area vectors (US@X)

45 12.29% user variables

113 30.60% code definitions

1  0.27% vocabulary

1  0.27% tickles